### SafeSenior – Hardware Technical Description

### Overview

This firmware transforms a Wi-Fi-enabled Arduino, equipped with a button, two LEDs, and a buzzer, into a connected emergency alert device.

The system establishes a wireless connection, authenticates with the backend using a stored device identifier, sends SOS events when the button is pressed, polls the backend for updates (for example, “help on the way”), and preserves its credentials.

Through these mechanisms, the Arduino operates as a fully autonomous IoT client within the SafeSenior ecosystem, interacting directly with the Flask API and Supabase database.

#### Include Section

#include <WiFi.h>

#include <HTTPClient.h>

#include <EEPROM.h>

#include <ArduinoJson.h>

The included libraries provide the necessary capabilities:

- WiFi.h – Allows the Arduino to connect to a Wi-Fi network and communicate with remote servers.

- HTTPClient.h – Wnables HTTP GET and POST operations.

- EEPROM.h – Provides access to non-volatile memory, allowing the device to retain stored information across restarts.

- ArduinoJson.h – Allows efficient parsing of JSON responses returned by the backend.

#### Configuration Constants

const char* WIFI_SSID     = "6G";

const char* WIFI_PASSWORD = "netdogabi";

const char* API_BASE_URL  = "https://your-flask-api.vercel.app";

const char* LOGIN_ENDPOINT = "/devices/login";

const char* SOS_ENDPOINT   = "/sos";

const char* NOTIF_ENDPOINT = "/notifications";

These constants define the operating environment:

- Wi-Fi credentials are used for network association.

- The base URL and related endpoints specify how the device interacts with the Flask API.

Once the Arduino establishes network connectivity, all communications to the backend follow these route definitions.

#### Hardware Pins

#define BUTTON_PIN  2

#define RED_LED     8

#define BLUE_LED    9

#define BUZZER_PIN  7

- Button (pin 2) – Controls the initiation and termination of SOS events.

- Red LED (pin 8) – Indicates that the SOS mode is currently active.

- Blue LED (pin 9) – Indicates that help has been acknowledged by another user.

- Buzzer (pin 7) –Produces a brief tone whenever an SOS event is activated.

This set of indicators provides clear physical feedback about the current system state.

#### Global Variables

bool sosActive = false;

String deviceId = "";

String jwtToken = "";

unsigned long lastPoll = 0;

const unsigned long POLL_INTERVAL = 10000;

const int EEPROM_SIZE = 512;

These variables maintain runtime context:

- sosActive records whether the device believes an SOS is ongoing.

- deviceId holds the identifier assigned to the device at registration.

- jwtToken stores the JSON Web Token used for authenticated requests.

- lastPoll and POLL_INTERVAL control periodic checks for notifications.

- EEPROM_SIZE defines available non-volatile memory.

These values allow the Arduino to remain synchronized with the backend while performing tasks autonomously.

#### Connecting to Wi-Fi

void connectWiFi() {

Serial.print("Connecting to Wi-Fi");

WiFi.begin(WIFI_SSID, WIFI_PASSWORD);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");}

Serial.println("\nWi-Fi connected!");}

At startup, the Arduino attempts to connect to the configured Wi-Fi network.
It loops until a connection is established, after which it can interact with backend endpoints.

#### EEPROM Management

##### Saving Data

void saveStringToEEPROM(int startAddr, const String& str)

This function writes the provided string into EEPROM, ensuring it persists after reboot or power loss. This mechanism is used to store the device ID and the authentication token.

##### Reading Data

String readStringFromEEPROM(int startAddr)

This function restores strings previously saved to EEPROM. On startup, the Arduino retrieves both the stored device ID and the latest valid token. EEPROM usage prevents the need for repeated pairing or manual configuration.

#### Device Authentication

bool getDeviceToken() {

if (WiFi.status() != WL_CONNECTED) return false;

...

String body = "{\"device_id\":\"" + deviceId + "\"}";

int code = http.POST(body);

The Arduino authenticates by sending its unique identifier to the backend’s /devices/login endpoint. The backend validates the identifier and responds with a temporary JWT.

jwtToken = doc["token"].as<String>();

saveStringToEEPROM(100, jwtToken);

The token is then stored in memory for immediate use and saved in EEPROM, so it persists across restarts. This token allows the Arduino to access protected endpoints securely.

#### SOS Event Handling

void toggleSOS() {

if (WiFi.status() != WL_CONNECTED || jwtToken == "") return;

...

int code = http.POST("{}");

Pressing the button triggers a POST request to /sos. The backend determines whether to start or stop the SOS event based on current context. When an SOS becomes active, the firmware updates physical indicators:

sosActive = response.indexOf("\"active\":true") > 0;

digitalWrite(RED_LED, sosActive);

if (sosActive) tone(BUZZER_PIN, 500, 250);

If the SOS is stopped, indicators revert accordingly.

#### Notification Polling

void checkNotifications() {

if (WiFi.status() != WL_CONNECTED || jwtToken == "") return;

int code = http.GET();

Every defined interval (10 seconds by default), the Arduino queries the backend for updates. If a notification indicating “help on the way” is received, the blue LED is enabled:

digitalWrite(BLUE_LED, payload.indexOf("help_on_the_way") > 0 ? HIGH : LOW);

This ensures the device visually reflects backend-driven state changes.

#### Setup Routine

void setup() {

Serial.begin(9600);

EEPROM.begin(EEPROM_SIZE);

...

connectWiFi();

During setup:

- Serial communication and EEPROM are initialized.

- GPIO pins are configured.

- Wi-Fi connection is established.

- Device ID and stored token are retrieved from EEPROM.

- If the device ID is missing, the Arduino waits for it to be entered through the serial monitor, then stores it permanently.

- If no valid token is detected, the device requests a new one from the backend.

After setup completes, the Arduino is authenticated and ready to interact with the backend.

#### Main Execution Loop

void loop() {

static int lastButton = HIGH;

int current = digitalRead(BUTTON_PIN);

if (current == LOW && lastButton == HIGH) {

toggleSOS();

delay(300);

}

lastButton = current;

The loop continuously performs two key tasks:

- Monitoring Button Input – Detects transitions indicating a button press.
When detected, the SOS state is toggled.

- Polling the Backend – At each interval defined by POLL_INTERVAL, the Arduino requests the current notification list. The physical indicators are updated accordingly.

This ensures real-time responsiveness both to local input and remote events.

#### Internal Operation Summary

- Startup

  - Connects to Wi-Fi.

  - Loads stored identifiers and tokens.

  - Authenticates if necessary.

- Idle State

  - LEDs and buzzer remain inactive.

  - Polls backend periodically for updates.

- SOS Activation

  - Button press triggers a request to /sos.

  - Backend creates an SOS event and notifies connected users.

  - Red LED and buzzer activate.

- Help Acknowledgment

  - Connected users respond through their applications.

  - The next polling cycle detects the “help on the way” state.

  - Blue LED activates.

- SOS Termination

  - Another button press stops the SOS.

  - Indicators are cleared, and backend state is updated.

#### Operational Principle

Once authenticated, the Arduino maintains a persistent association with the backend and regularly synchronizes its local state with the server. This ensures that emergency events and acknowledgments are consistently reflected on both the physical device and the connected application, without requiring manual resets or configuration changes.

| Component | Role |
| --- | --- |
| Arduino Wi-Fi Device | Executes local logic, handles physical inputs, and communicates with backend |
| Flask API | Manages authentication, controls SOS lifecycle, propagates notifications |
| Supabase Database | Stores persistent records such as users, connections, devices, SOS events, notifications |
| Android Application | Provides user interaction and relays acknowledgments |